Smart Parking via Distributed Vision Version 3.0 11/28/2017

This project utilizes a camera-based system to provide an effective solution for general parking area/garages. The project combines the physical sensors, microcontrollers, smartphones and dedicated software to demonstrate the fundamental idea of IoT (Internet of Things). Our goal is to monitor the available parking space and update the real-time data on the Amazon Web Service (AWS) to allow the users to access the information on their smartphone.

Installing Raspbian on Raspberry Pi:

  - Download Image from https://www.raspberrypi.org/downloads/

  - Unzip .zip downloads to get the image file (.img) to write to a SD card.

  - Download Etcher and install the etcher packages. https://etcher.io/

  - Connect an SD card reader with the SD card inside.

  - Choose 'Flash!' to begin writing data to the SD card.

  - NOOBS is easy to implement. The link is on below. https://www.raspberrypi.org/documentation/installation/noobs.md

Connect Raspberry Pi by PuTTY and Ethernet cable:

- Download PuTTY from http://www.putty.org/

- Download Advance IP Scanner https://www.advanced-ip-scanner.com/

- Switch off the Raspberry Pi board

- Disconnect the motors and the batteries.

- Make sure the SD card is in the Raspberry Pi.

- Plug in my Wi-Fi dongle to a USB port on the Raspberry Pi.

- Connect my Ethernet cable to my computer and to the Raspberry Pi.

- Plug in a power adapter into the Raspberry Pi.

- Power on the Raspberry Pi.

- Go Control Panel.

- Click Network and Internet.

- Click Network Connections.

- Click the internet source to share network in sharing properties.

- Go Ethernet and click packets.

- Open Advance IP Scanner search Raspberry Pi.

- Open Putty and enter the Host Name as the IP address of Raspberry Pi

- Login PuTTY and

- Turn On VNC server or Teamviewer on the Raspberry Pi

- Go VNC viewer or Teamviewer on my laptop.

- Type IP and port to connect Raspberry Pi.

Hardware components & Drivers:

Raspberry Pi 3 Model B Starter kit (including Raspberry Pi 3 Model B, power adaptor, 32G micro SD card

https://www.amazon.com/CanaKit-Raspberry-Complete-Starter-Kit/dp/B0778CZ97B/ref=sr_1_1_sspa?s=pc&ie=UTF8&qid=1511920261&sr=1-1-spons&keywords=raspberry+pi+3&psc=1

Raspberry Pi Camera Board, Version 2.1

http://www.newark.com/raspberry-pi/rpi-8mp-camera-board/camera-board-8-mp-raspberry-pi/dp/77Y6521

Camera Lens to replace the original Raspberry Pi Camera lens

http://www.m12lenses.com/16-0mm-F2-0-Mega-Pixel-CCTV-Board-Lens-p/pt-1620bmp.htm

Lidar-Lite V3 https://www.sparkfun.com/products/14032?gclid=CjwKCAiAr_TQBRB5EiwAC_QCq-xb02z_Mkq-JKdIUzdnb67QI5LNe9PftPCVMAV9vD1Ck940zg1RgBoCssgQAvD_BwE

HS-422 Servo Motor

http://www.robotshop.com/ca/en/hitec-hs422-servo-motor.html

Servo HAT for Raspberry Pi

https://www.adafruit.com/product/2327

Bracket and frames to hold Lidar

https://www.superdroidrobots.com/shop/item.aspx/lidar-lite-mounting-bracket/2324/

Assemble the Hardware components

- Install the bracket for holding the Lidar on the Servo Motor

- The Servo HAT can be mounted onto the Raspberry Pi module.

- Connect the Lidar sensor and Pi using I2C interface

- Connect the Servo motor and Pi using PWM pin on the Servo HAT (A 9v battery is needed to power the Servo HAT)

- Replace the original Raspberry Pi Camera with new lens. Tutorial Link: http://wiki.raspberrytorte.com/index.php?title=Camera_Module_Lens_Modifcation

- Connect Raspberry Pi Camera to the Raspberry Pi module

Software required in the Raspberry Pi

I2C interface for the Raspberry Pi and Servo HAT.

https://learn.adafruit.com/adafruit-16-channel-pwm-servo-hat-for-raspberry-pi/

when the Servo HAT is connected, you should see the following. The device address is 0x40

I2C interface for the Raspberry Pi and Lidar Lite V3. The device address is 0x62

.

Library for Lidar Sensor: https://github.com/Sanderi44/Lidar-Lite

Camera Installation, Image IO packages, OpenCV & OpenALPR:

Raspberry Pi camera installation

  - Connect the camera module into the Raspberry Pi’s camera port.

  - Reference: https://www.youtube.com/watch?v=GImeVqHQzsE

  - Enable my camera module by using the Raspberry Pi command.

$ sudo raspi-config

  - Reboot my Raspberry Pi

  - Test out the camera module

$ raspistill -o output.jpg

Developer tools & Image I/O packages

  - All of the instructions are followed by the website https://www.pyimagesearch.com/2016/04/18/install-guide-raspberry-pi-3-raspbian-jessie-opencv-3/

  - Command terminal shown below to update the Raspberry Pi

  - Install the developer tools and packages such as Cmake.

  - Install the image I/O packages such as jpeg, tiff, jasper, and png.

  - Install the GTK development library

  - Install the video I/O packages such as avcodec and avformat.

  - Install the libraries used to optimize the operations within OpenCV.

  - Install pip.

  - Install virtualenv and virtualenvwrapper (Optional)

Update Raspberry Pi profile after installing Virtual Environment (Optional)

  - Update Profile

  - Reload Profile

  - Create computer vision virtual environment named cv

OpenCV Installation

  - Download OpenCV and unpack packets

From http://sourceforge.net/projects/opencvlibrary/files/opencv-unix/2.4.10/opencv-2.4.10.zip/download

However, our team has used the latest versions of OpenCV (Version 3.2)

Therefore change 2.4.10.zip into 3.2.0.zip and download from https://github.com/Itseez/opencv/archive/3.2.0.zip

  - Setup the build

  - Command:

$ cmake -D CMAKE_BUILD_TYPE=RELEASE -D CMAKE_INSTALL_PREFIX=/usr/local -D BUILD_NEW_PYTHON_SUPPORT=ON -D INSTALL_C_EXAMPLES=ON -D INSTALL_PYTHON_EXAMPLES=ON  -D BUILD_EXAMPLES=ON ..

  - Install OpenCV

    - Compile and install OpenCV into the Raspberry Pi with 4 CPUs

    - Compile and install OpenCV into the Raspberry Pi with a simple CPU

  - Locate the OpenCV in /usr/local/lib/python2.7/site-packages

However, our team uses python 3.4. Therefore, replace the python2.7 to python3.4 because they are not sharing the same packages.

  - Test the installation of OpenCV by using Python

  - Example of using OpenCV

    - Import a OpenCV library into the Python program

    - Call the functions included in OpenCV

      - For example, cv2.cvtColor.

Installing PiCamera Library

  - Activate the virtual environment (Optional)

  - install PiCamera by utilizing pip

  - Example of using PiCamera

Installing NumPy on my Raspberry Pi

  - Using pip command to install numpy packa

OpenALPR Implementation

Our team has followed the compilation instructions of the OpenALPR installation. The following link shows how the installation can be processed in Linux or Raspbian OS.

https://github.com/openalpr/openalpr/wiki/Compilation-instructions-(Ubuntu-Linux)

  - Install prerequisites of OpenALPR installation such as libopencv-dev and libtesseract-dev.

$ sudo apt-get install libopencv-dev libtesseract-dev git cmake build-essential libleptonica-dev

$ sudo apt-get install liblog4cplus-dev libcurl3-dev

  - Install beanstalkd library in the Raspberry Pi

$ sudo apt-get install beanstalkd

  - Get the latest code of OpenALPR from GitHub

$ git clone https://github.com/openalpr/openalpr.git

  - Setup a compile environment for OpenALPR installation

$ cmake -DCMAKE_INSTALL_PREFIX:PATH=/usr -DCMAKE_INSTALL_SYSCONFDIR:PATH=/etc ..

  - Compile the library

$  make –j4

$  sudo make install

  - Example of using OpenAlpr library on Python

AWS IoT and DynamoDB Setup:

- Set Up AWS IoT. Follow the official developing guide from Amazon. http://docs.aws.amazon.com/iot/latest/developerguide/what-is-aws-iot.html

- Setup an AWS account and AWS IoT service.

- Register the things (Cameras) and set the rules http://docs.aws.amazon.com/iot/latest/developerguide/register-device.html

- Set a rule that atomically pushed the message from IoT device to DynamoDB, REMEMBER to use DynamoDBv2.

- Create a table on DynamoDB follow the instruction or the guideline from the AWS IoT console. http://docs.aws.amazon.com/amazondynamodb/latest/developerguide/SettingUp.DynamoWebService.html

- Try the MQTT simulator on AWS IoT console to publish some fake sensor data or license data to your own assigned topic and double check if the data is being stored in the database. http://docs.aws.amazon.com/iot/latest/developerguide/config-and-test-rules.html

AWS IoT SDK and Run code on Raspberry Pi:

- Prerequisite: Install OpenCV and other dependences; setup all hardware drivers and libraries.

- Download the AWS IoT SDK from Amazon and follow the official guide to set the SDK on Pi  http://docs.aws.amazon.com/iot/latest/developerguide/iot-sdk-setup.html & https://github.com/aws/aws-iot-device-sdk-python/blob/master/README.rst

- After that, please save the credentials from AWS to the Pi.

- Replace the credentials or reset the default credentials in sample code.

- Run the code on CMD by typing Final.py. Note please enter the relevant filed

python3 FileName.py -endpoint –root_CA -key -cert –topic

- Note: Please change the parameters and constants in the code such as positions, distances, and angels based on your measurement of your own testing environment. You might also need to change or add control flows of the program as well to get the best result in different situation.